ONT Model no     : OP-XONT 71110N

Firmware version: XONT71110N_OPTILINK_BSNL_1_1.0.24-200226.tar

Upgrade Process for TIP partner or operator:

- Download the Firmware patch from BSNL website.

- Extract the firmware patch from .rar to .tar.   (install WINRAR software in laptop/system)

- Connect Laptop/system directly to ONT LAN port. (ONT IP address: 192.168.10.1)

- Assign IP address of same subnet in laptop/system LAN card (Example: 192.168.10.200)

- Open the ONT IP 192.168.10.1 on web browser.

(ONT management login credentials: Username : admin ; Password: optilink)

- Click on the management tab and check for the upgrade tab.

- Click on the upgrade and select the firmware path and click on upgrade.

- Upgrade process starts. Keep the ONT ping open on the screen parallel.

- It will take about approx 3 minutes for the ONT to get upgraded.

- After successful upgradation, ONT will reboot. Check for the ONT reachability (ping 192.168.10.1)

- Again open the ONT on web interface and login with default credentials

(Username : admin ; Password: optilink ; CATPCHA: xxxxxx

- The Page will be redirected to change the default password.

- Here change the password and re-login ONT with new credentials.

Important Note:

- Attached File is compressed in ‘.rar’ file to protect the software from being corrupt.

- Extract the ‘.rar’ file from winrar software and use the firmware file ‘.tar’ for up-gradation.